| Unit 1: Getting Started with TI-Innovator™ Hub | Application: Traffic Light |  |
| --- | --- | --- |
| Write a program that controls a traffic light. | Objectives: |  |
|  | - Control the COLOR LED to simulate a traffic light using a single bulb - Create a sequence of statements with proper timing controls |  |
| Your task is to write a program that controls a traffic light. The light will be simulated using the COLOR LED on the TI-Innovator Hub. The COLOR LED should switch from green to yellow to red AND from red to yellow to green. Timing is up to you. Your program will have a sequence of statements that simulate the change from RED to GREEN to YELLOW to RED. A sequence control structure in programming is a set of statements that are processed one after another, from top to bottom, without interruption. |  |  |
| Using Text as a pause The Disp command displays a message on the HOME screen of the calculator. It can be used to display the value of a variable as in Disp X, or it can display a string. But it does not stop the program from continuing. The Text statement on the I/O menu displays a dialog box and waits for the user to press enter or click the ‘OK’ button before processing the rest of the program. |  |  |
| Setting up the title screen - Begin a new program, and call it APPLIC1. - Get the Text keyword from the I/O menu. - In quotation marks, add the message “Traffic Light <enter> Press enter…” as shown. |  |  |
| Set the Colors First, we set the color to red by sending the RGB values of 255 0 0. In the example on the right, we use a Wait statement to tell the calculator to wait 5 seconds before sending the next command to the TI-Innovator Hub. The red light will stay on during this time. Your task is to add the statements to make the light green, then yellow, then red again. Challenge: Add SOUNDs so that a blind person (pedestrian, not driver!) can tell which color the light is. |  |  |