With your Rocket partners, do the following.

- Locate the launch of your rocket. There should be two views, Close Up, and Long Shot.

- Place at least one of these video clips as a file on the computer. Use the Video Capture software. Trim the video clip to have just the part for your launch.

- Save the file in the S-drive, in the Working folder.

- Determine the scale for the rocket using the Scaling Clips, or some other reference.

- Open your video in VideoPoint. Scale the object, relocate the Origin to a more appropriate place, digitize its motion, and give the Points the mass of the rocket.

- Determine the following:

| Measure | Value |
| --- | --- |
| Rocket final velocity at beginning of flight, i.e. at end of video clip (m/s) |  |
| Rocket acceleration at beginning of flight (m/s2) |  |
| Rocket average velocity for the part on the video (m/s) |  |
| Maximum height of the rocket, assuming the velocity at the end of the video is after the fuel is spent in the engine. (m/s) |  |
| Other Measurement of Interest. |  |